Script (entropy_detection.py)

from pox.core import core

import pox.openflow.libopenflow_01 as of

import math

from collections import defaultdict

from pox.lib.recoco import Timer

log = core.getLogger()

class EntropyDetection:

def __init__(self, interval=5):

self.src_ip_counts = defaultdict(int)

self.dst_ip_counts = defaultdict(int)

self.total_packets = 0

self.entropy_threshold = 3.5  # Adjust based on network behavior

# Run entropy calculation periodically

Timer(interval, self._calculate_entropy, recurring=True)

def _handle_packet_in(self, event):

packet = event.parsed

if not packet.parsed:

return

# Extract source and destination IPs

ip = packet.find('ipv4')

if ip:

self.src_ip_counts[ip.srcip] += 1

self.dst_ip_counts[ip.dstip] += 1

self.total_packets += 1

def _calculate_entropy(self):

src_entropy = self._calculate_shannon_entropy(self.src_ip_counts)

dst_entropy = self._calculate_shannon_entropy(self.dst_ip_counts)

if src_entropy > self.entropy_threshold and dst_entropy < 1.5:

log.info("Potential SYN Flood Detected! High Source IP Entropy: {:.2f} | Low Destination IP Entropy: {:.2f}".format(src_entropy, dst_entropy))

else:

log.info("Source IP Entropy: {:.2f} | Destination IP Entropy: {:.2f}".format(src_entropy, dst_entropy))

# Reset counters for next interval

self.src_ip_counts.clear()

self.dst_ip_counts.clear()

self.total_packets = 0

def _calculate_shannon_entropy(self, counts):

total = sum(counts.values())

if total == 0:  # No packets received, return entropy 0

return 0

entropy = -sum((count / float(total)) * math.log(count / float(total), 2)

for count in counts.values() if count > 0)

return entropy

def launch():

log.info("Entropy-Based DDoS Detection Module Started")

detection = EntropyDetection()

core.openflow.addListenerByName("PacketIn", detection._handle_packet_in)

How to Run in POX

1️- Save the script in:

pox/ext/entropy_detection.py

2️- Run POX with the module

./pox.py forwarding.l2_learning entropy_detection

📌 Explanation of the entropy_detection.py Script (Line-by-Line)

This script is designed to detect SYN flood DDoS attacks using entropy-based analysis of Source IP and Destination IP distributions.

1️- mport Required Modules

from pox.core import core

import pox.openflow.libopenflow_01 as of

import math

from collections import defaultdict

from pox.lib.recoco import Timer

- pox.core: Core POX framework for SDN event handling.

- pox.openflow.libopenflow_01: Library to interact with OpenFlow version 1.0.

- math: Used for computing entropy using logarithms.

- collections.defaultdict: A dictionary that provides default values for missing keys.

- pox.lib.recoco.Timer: POX's built-in timer module for periodic tasks.

2️- nitialize Logging

log = core.getLogger()

- Creates a logger to display messages in the POX terminal.

3️- Define the EntropyDetection Class

class EntropyDetection:

- This class handles entropy calculations and attack detection.

4️- nitialize Variables & Start Timer

def __init__(self, interval=5):

self.src_ip_counts = defaultdict(int)

self.dst_ip_counts = defaultdict(int)

self.total_packets = 0

self.entropy_threshold = 3.5  # Adjust based on network behavior

# Run entropy calculation periodically

Timer(interval, self._calculate_entropy, recurring=True)

- self.src_ip_counts: Dictionary storing counts of Source IPs.

- self.dst_ip_counts: Dictionary storing counts of Destination IPs.

- self.total_packets: Tracks the total number of packets received.

- self.entropy_threshold: If entropy exceeds this value, it indicates a possible attack.

- Timer(interval, self._calculate_entropy, recurring=True):

  - Runs _calculate_entropy() every 5 seconds.

5️- Handle Incoming Packets (_handle_packet_in)

def _handle_packet_in(self, event):

packet = event.parsed

if not packet.parsed:

return

- This function listens for OpenFlow "PacketIn" events.

- packet = event.parsed: Extracts the packet details from the event.

- if not packet.parsed: Ignores malformed packets.

6️- Extract Source & Destination IPs

ip = packet.find('ipv4')

if ip:

self.src_ip_counts[ip.srcip] += 1

self.dst_ip_counts[ip.dstip] += 1

self.total_packets += 1

- packet.find('ipv4'): Checks if the packet is an IPv4 packet.

- If it's an IPv4 packet:

  - Increases the count of the Source IP.

  - Increases the count of the Destination IP.

  - Increments total_packets.

7️- Compute Entropy Periodically (_calculate_entropy)

def _calculate_entropy(self):

src_entropy = self._calculate_shannon_entropy(self.src_ip_counts)

dst_entropy = self._calculate_shannon_entropy(self.dst_ip_counts)

- Computes Shannon entropy for Source IPs and Destination IPs.

8️-Detect DDoS Attacks

if src_entropy > self.entropy_threshold and dst_entropy < 1.5:

log.info("Potential SYN Flood Detected! High Source IP Entropy: {:.2f} | Low Destination IP Entropy: {:.2f}".format(src_entropy, dst_entropy))

else:

log.info("Source IP Entropy: {:.2f} | Destination IP Entropy: {:.2f}".format(src_entropy, dst_entropy))

- If Source IP entropy is high and Destination IP entropy is low, it indicates a possible DDoS attack.

- Otherwise, just logs entropy values.

9️- Reset Counters for Next Interval

self.src_ip_counts.clear()

self.dst_ip_counts.clear()

self.total_packets = 0

- Clears IP count data every 5 seconds.

🔟 Compute Shannon Entropy

def _calculate_shannon_entropy(self, counts):

total = sum(counts.values())

if total == 0:  # No packets received, return entropy 0

return 0

entropy = -sum((count / float(total)) * math.log(count / float(total), 2)

for count in counts.values() if count > 0)

return entropy

- Formula: H(X)=−∑P(x)log⁡2P(x)H(X) = -\sum P(x) \log_2 P(x)

- total = sum(counts.values()): Sum of all packets.

- If total == 0, returns 0 entropy.

- Otherwise, calculates entropy using the Shannon formula.

🔟 Register the POX Module

def launch():

log.info("Entropy-Based DDoS Detection Module Started")

detection = EntropyDetection()

core.openflow.addListenerByName("PacketIn", detection._handle_packet_in)

- Runs EntropyDetection when POX starts.

- Attaches _handle_packet_in() to OpenFlow PacketIn events.

📌 Summary of How It Works

1️⃣ Monitors incoming packets
2️⃣ Counts unique Source IPs & Destination IPs
3️⃣ Calculates entropy every 5 seconds
4️⃣ Detects high Source IP entropy (DDoS attack)
5️⃣ Logs results

🛠 How to Run in POX

1️- Save the script in:

pox/ext/entropy_detection.py

2️- Run POX with the module

./pox.py forwarding.l2_learning entropy_detection

🚀 Final Thoughts

- ✅ Detects SYN Flood attacks using entropy.

- ✅ Fully automated monitoring.

- ✅ Easily expandable for other DDoS attacks.

Let me know if you need any modifications! 🚀

No, this script is not using joint entropy; it is only computing the Shannon entropy of individual features (Source IPs and Destination IPs) separately. Joint entropy involves measuring the relationship between two variables together, not independently.

📌 Difference Between Shannon Entropy and Joint Entropy

- Shannon Entropy (used in the script):

H(X)=−∑P(x)log2P(x)

  - Computes entropy for one feature at a time (e.g., Source IPs or Destination IPs).

  - If H(Source IPs) is high, it means there are many unique Source IPs (indicating spoofing).

  - If H(Destination IPs) is low, it means traffic is targeting one victim (common in DDoS attacks).

- Joint Entropy (required for joint entropy-based DDoS detection):

H(X,Y)=−∑P(x,y)log2P(x,y)
Measures how two features (Source IPs and Destination IPs) interact together.

  - Instead of treating them separately, it detects patterns in how different IPs communicate.

  - More effective for DDoS attack detection, as it captures complex correlations between Source and Destination IPs.

📌 How to Modify the Script for Joint Entropy

Instead of calculating two separate entropy values, we need to calculate one joint entropy value for Source IP + Destination IP pairs.

1️⃣ Modify Data Collection

Replace this:

self.src_ip_counts[ip.srcip] += 1

self.dst_ip_counts[ip.dstip] += 1

With this:

self.ip_pair_counts[(ip.srcip, ip.dstip)] += 1

This stores both Source and Destination IPs together.

2️⃣ Modify the Entropy Calculation

Replace:

src_entropy = self._calculate_shannon_entropy(self.src_ip_counts)

dst_entropy = self._calculate_shannon_entropy(self.dst_ip_counts)

With:

joint_entropy = self._calculate_shannon_entropy(self.ip_pair_counts)

This ensures we calculate entropy based on Source-Destination pairs, instead of treating them separately.

3️⃣ Modify the DDoS Detection Logic

Instead of:

if src_entropy > self.entropy_threshold and dst_entropy < 1.5:

Use:

if joint_entropy > self.entropy_threshold:

- If joint entropy increases, it suggests many different sources targeting one victim → DDoS detected.

📌 Summary of Changes

| Feature | Current Script | Joint Entropy-Based Detection |
| --- | --- | --- |
| What is measured? | Source IP entropy & Destination IP entropy separately | Entropy of Source-Destination pairs together |
| Detection Logic | High Source IP entropy + Low Destination IP entropy | High Joint Entropy |
| DDoS Indicator | Many different source IPs targeting one | Many random connections happening together |
| Effectiveness | Can detect spoofing but might miss multi-target attacks | More accurate for DDoS patterns |

🚀 Next Steps

🔹 Do you want me to modify the script and give you the full code with joint entropy?